Project 2   -     part 3   -    Wrapper

Hadeer Ghoneim

Verification Diploma

• Verification plan

• UVM Diagram

o How UVM testbench work

- Randomized Stimulus Generation
Generates protocol-compliant and corner-case SPI transactions.

- Golden Model Comparison
C-based reference model ensures cycle-accurate checking.

- Assertion-Based Verification
Validates protocol timing, FSM transitions, and data integrity.

- Coverage Analysis

  - Functional Coverage: SPI states, RAM operations, and cross-coverage

  - Code Coverage: line, branch, expression

  - Assertion Coverage: protocol timing checks

UVM Testbench Architecture

Env  → UVM Environment

test → UVM Testbench

agent  → UVM Agent containing:

sequencer → Generates sequences

Driver  → Drives stimulus to DUT

Monitor → Observes and collects transactions

Sequence → Defines test sequences for different SPI operations

Sequence item → Defines transaction-level items for SPI communication

config → Configuration settings for SPI testbench

interface → UVM Interface to connect DUT with the UVM testbench

scoreboard → Compares actual DUT output with expected results

coverage → Defines and monitors functional coverage metrics

Top → top module

• Code Coverage report, Functional Coverage report, and Sequential Domain Coverage report

• Bug report

SPI_slave Bugs Summary:

Critical Bugs:

- State mixup: READ_ADD and READ_DATA states swapped in FSM logic

- Missing counter reset: Counter not initialized in IDLE state

- MISO timing: Transmission depends on tx_valid instead of SPI clock

- Bit width inconsistency: 8-bit vs 10-bit transfers in read operations

Solu:

Critical Fixes:

- Swap states: READ_ADD ↔ READ_DATA in CHK_CMD logic

- Initialize counter: Add counter <= 10; in IDLE state

- Fix MISO timing: Remove tx_valid dependency, transmit continuously in read states

- Standardize bit width: Use consistent 10-bit transfers throughout

Major Fixes:

- Add default case: default: ns = IDLE; in state machine

- Fix rx_valid: Clear in all states except when data is ready

- Improve flag management: Reset received_address in IDLE state

- Complete reset: Ensure all registers properly initialized

Minor Fixes:

- Use blocking assignments properly in combinational logic

- Add output register for MISO to prevent glitches

- Fix counter reset: Always reset to 10 for consistency

Wrapper bugs

Signal Width Mismatch - 10-bit rx_data connected to 8-bit RAM port

Missing RAM Address - No address extraction from 10-bit frame

• QuestaSim waveform snippets

• Assertions Table